Hi everyone, my name is Delaney! I work on the Shadow Scope project to create educational materials and activities to go along with our scope. However, right now, we are going to be talking about how to build your own shadow scope at home for under $30! I am going to walk you through my first attempt at building a shadow scope at home, so that you all can learn from my successes and failures and streamline the process if you want to build your own   let’s get started!

To start out, let’s talk about the supplies that are required in order to build a scope:

- A webcam with USB connectivity

  - This allows us to visualize our samples on the computer screen

- A container that could go over the webcam and create a dark environment

  - Needs to be made of material that can have a pinhole poked into it)

- Silicone

  - Used to seal the light sensor of the webcam so it doesn’t get damaged from looking at water samples

- A flashlight

  - Preferably with a single LED so that our light is concentrated

- Small flathead and Philips screwdrivers

- Tape

- Scissors

- Thumbtack

- Pencil

Seems pretty straight forward, right?

I wanted to try to build this scope with supplies that I could find at local stores, so I could avoid having to wait for my items to get delivered. So for that reason, I started my journey at place that seems to have everything: Walmart.

So, I headed off to my local Walmart (it was a nice snowy day) and started looking around for the supplies I needed!

I began my search in the Paint & Hardware section, looking for Silicone to help keep the internal components of the webcam sealed away from any water samples we were going to look at with the at-home scope. I was able to find a good tube for only $3.77! Really you are looking for one that is waterproof and is safe to use indoors.

After the silicone success, I went over to Sporting Goods to find a flashlight. There was one with a single LED that was B-R-I-G-H-T and only cost $1… perfect!

Now, this is where my search got a little tougher… I wandered over to the Technology department in the Computer Accessories isle to look for a cheap webcam that I could disassemble for the shadow scope. However, the cheapest I could find was a $27 webcam, which wasn’t going to work, because that would have cost almost the entire $30 budget. So, that was not going to work, and I planned to do some online research for where I could purchase a cheaper webcam in-store after I completed the rest of my shopping!

After the disappointment with the webcam, I looked for a container that could cover the webcam to create the dark environment we need for the shadow scope. I was specifically looking for one of the cylindrical frozen limeade cans with aluminum tops because I felt this would be the perfect size and material for what we need. Plus, limeade would make a perfect refreshment while building the scope  Yet again, though, Walmart let me down. There were no frozen limeade cans in sight. So, I decided to cut my losses ant purchased the silicone and the flashlight for a grand total of $5.56 (with tax).

At this point, I knew I would have to do some research about where I could pick up a webcam, but I knew for sure that my local grocery store would have the limeade cans that I needed, so I headed that way! I entered the frozen fruit section, and immediately found what I was looking for… LIMEADE! It’s the little wins, right? The total came out to $1.99, putting me at a $7.55 grand total so far. That left me with around $22.00 to purchase a webcam! Research time!

Turned out that a USB-enabled, 1080p (720p would work too) webcam that was in stores for less than $22 was harder than expected to come by.. but I did find a few options:

BigLots $11 (sale) Webcam Link		Microcenter $5 Webcam Link		Microcenter $10 (sale) Webcam Link

I decided to go with the $10 Microcenter option because it was available in-store and was close-by. The other two cameras would have worked too, keep an eye on Mac vs. PC compatibility depending on what type of computer you have!

I went to microcenter and purchased the camera which rang up to $10.87. That brought my grand total to $17.55!

I had the remaining supplies already available to me at home, so I called it quits on my shopping spree and went hope to start building! You can find out how to build the shadow scope in part 2!